Request for Proposal (RFP) For MDF/IDF Cable Management and Fiber Run Project Unified School District No. 261, Sedgwick County, Kansas (Haysville)

| Proposal Issued Date: Wednesday, October 7, 2026 |  | Proposal Due Date: Friday, November 27, 2026 at 11:59 pm CST |  | Proposals Submitted To: https://www.crwconsulting.com/ifcb/ifcb-drfdetail/TkRFeU5UWXg= |
| --- | --- | --- | --- | --- |

Submit RFP questions no later than Wednesday, October 28, 2026, at 11:59 pm CST via the website at: https://www.crwconsulting.com/ifcb/ifcb-drfdetail/TkRFeU5UWXg=

All bids must be submitted to https://www.crwconsulting.com/ifcb/ifcb-drfdetail/TkRFeU5UWXg= (login as a vendor, choose the district/IFCB you are interested in, then select upload a question or submit a bid)

District Information

Unified School District No. 261, Sedgwick County, Kansas (Haysville) – hereinafter referred to as the District - primarily serves the communities of south Wichita and Haysville. The District has approximately 5200 students and 1100 staff spread across 16 buildings, 14 of which are included in this project.

District Facilities

A complete listing of District facilities to be included in this project can be found in Attachment A.

Project Overview

The District is seeking a vendor to clean up the existing Cat5, Cat6, and fiber cabling in 42 MDF and IDF closets located across 14 buildings. Work will include removing old patch panels and termination points, installing new patch panels, and reterminating existing Cat5, Cat6, and fiber cabling inside closets. The vendor will also install new wall plates and jacks and reterminate the existing cabling at applicable classroom and office locations. All applicable cabling shall be labeled, and cabling terminated or reterminated at both ends shall be tested. Existing switches shall be reconnected to the new patch panels as specified in the Scope of Work. The project also includes replacing existing multimode fiber with single-mode fiber runs as identified in Attachment C.

E-Rate/USAC Funded Project Requirements

E-Rate/USAC funding will be used to help fund this project. The selected vendor must comply with all applicable USAC requirements, including obtaining a Service Provider Identification Number (SPIN). This RFP is limited to E-Rate-eligible equipment and services. Ineligible work shall not be included in proposal pricing. Any award under this RFP is contingent upon E-Rate funding approval and final approval by the USD 261 Board of Education. The District reserves the right to reduce or cancel the project without penalty if E-Rate funding is not approved or is less than anticipated.

E-Rate Consultant Information

The District will be utilizing an E-Rate consultant, CRW Consulting, to assist in the RFP process. All bids and RFP-related questions must be directed to their website, https://www.crwconsulting.com/ifcb/ifcb-drfdetail/TkRFeU5UWXg= .

District Mandatory Tour/Walkthrough

To participate in the RFP, the proposed vendor must attend a walkthrough of some of the District’s wiring closets on Wednesday, October 21, 2026 at 9:00 am CST or Tuesday, October 27, 2026 at 2:00 pm CST. The walkthrough will be located at The Learning Center, 150 Stewart, Haysville, KS 67060. Bids from vendors that do not attend one of the two mandatory walkthroughs will be disqualified.

Scope of Work

The District is seeking proposals for work in 42 MDF and IDF wiring closets across 14 buildings. For purposes of this RFP, MDF and IDF are collectively referred to as closets, and Cat5, Cat6 and Fiber are collectively referred to as cabling. The specific work required for each closet is identified in Attachment B, which provides approximate quantities and the required scope by closet. The District will provide photographs of the wiring closets to assist vendors with preparing accurate proposals.

Work will include cable cleanup, rack and patch-panel replacement, cable retermination, labeling, testing, switch reconnection, cable management, and fiber optic cable replacement as identified in Attachment C. The vendor shall provide all labor, materials, equipment, testing, documentation, and disposal required to complete the identified work.

The project shall be substantially completed during the District’s summer break. Work may also be scheduled during evenings, nights, or weekends as required. Work shall be performed with no disruption to District network services during normal school and business hours. The vendor shall coordinate the work schedule and sequence of closets with District Technology staff before beginning work. Any required outage must be approved in advance by District Technology staff and performed outside normal operating hours.

- Photographs and maps: Photographs and maps of all closets & fiber runs will be provided at https://www.crwconsulting.com/ifcb/ifcb-drfdetail/TkRFeU5UWXg=.

- Cable cleanup and retermination: The vendor shall clean up and organize cabling in each wiring closet and replace racks and patch panels as required. Cat5 and Cat6 cabling included in the work shall be reterminated and labeled at both ends using the T568B wiring standard. At applicable classroom and office locations, the vendor shall replace existing wall plates and jacks with new wall plates and Cat6-rated jacks. Existing fiber cabling included in the work shall also be reterminated and labeled at both ends using LC connectors, except existing multimode fiber being replaced by a new fiber run identified in Attachment C. The only exceptions are individual cables specifically designated by District Technology staff for closet-end work only, which may include access points or other equipment located above ceilings or in difficult-to-access locations. For those cables, the vendor shall reterminate and label only the closet end and leave the equipment-end connection undisturbed. Tracing, reterminating, or labeling the equipment end is not required. Couplers are not allowed unless approved by District Technology. The District can provide a lift if needed. New or replacement Cat5 and Cat6 cable runs are not included in the base scope and shall not be performed without prior written approval from District Technology.

- Switch placement and patch cables: Relocate switches as necessary to place them directly above or below the associated 24-port patch panels. Use 6” slim 28 AWG Cat6 patch cables where switch and patch-panel placement allow. Reconnect switches using slim 28 AWG Cat6 patch cables rated for PoE Type 4. Use the shortest practical patch-cable length that allows proper routing, bend radius, and serviceability. Patch cable coloring should match color of cable patched down.

- Connection documentation and reconnection: Certain network connections are assigned to specific switch ports due to VLAN and other network configurations. Before disconnecting any network connection, the vendor shall identify, document, and label the cable, switch, and switch port. This includes connections that do not currently show an active link. After completing the work, return each connection to the same switch and switch port unless otherwise directed by District Technology staff.

- Cable labeling and documentation: The District will provide the required labeling scheme before work begins. The vendor shall label both ends of all cabling it terminates or reterminates, except cables designated for closet-end work only, which shall be labeled at the closet end only. For closet-end-only cables, the District will provide LLDP documentation for vendor to label cables with. Upon completion, the vendor shall provide the District with an electronic spreadsheet or similar document identifying the building, closet, cable identifier, patch-panel/port, switch/port, and destination when known. Unconfirmed destinations shall be identified as unverified. The vendor shall also provide documentation of all part manufacturers and part numbers.

- Testing and completion: The vendor shall test all cabling it terminates or reterminates at both ends. Cat5 and Cat6 cabling shall be tested for continuity and correct wiremap on all pairs. Fiber cabling shall be tested for continuity and end-to-end optical loss on all terminated strands. Certification, including OTDR certification, is not required. Vendor testing is not required for cables designated for closet-end work only. The vendor shall provide all required test results to the District. After completing the work and reconnecting equipment in each wiring closet, the vendor shall notify District Technology staff, who will verify network operation. District acceptance of each closet is contingent upon completion of all applicable scope requirements, including cable management, labeling, testing, documentation, rack and patch-panel work, and successful network verification. The vendor shall correct any termination, reconnection, or other deficiencies caused by its work at no additional cost before the District accepts the work in that closet.

- Workmanship warranty: The vendor shall warrant its workmanship for one year following District acceptance of the applicable work. During this period, the vendor shall correct defects caused by its installation or termination work at no additional cost to the District, including all necessary labor and materials.

- Fiber runs: Fiber runs & reterminations are identified in Attachment C. All new fiber cabling shall be 12-strand, OS2 single-mode fiber (9/125 µm), with all strands terminated using LC connectors at both ends. Cable shall be plenum rated. Fiber runs shall be continuous end-to-end with no intermediate splices unless specifically approved by District Technology. A minimum 20-foot service loop shall be neatly secured at each end of every new fiber run. The District can provide a lift if needed. Existing multimode fiber being replaced shall be disconnected, neatly coiled, and secured above the ceiling within the applicable wiring closet unless otherwise directed by District Technology.

- Disposal: The vendor shall place all removed materials in District-provided trash containers at each building.

- Materials and equipment: All materials and equipment supplied by the vendor shall be new, not used, refurbished, or gray market, and shall meet the District part specifications identified in Attachment E. The proposal must list manufacturer, model or part number, quantity, and unit price.

- Installation and restoration: All work shall follow manufacturer installation instructions and applicable codes. All wall-mounted racks shall be securely fastened to structural framing, masonry, concrete, or other suitable structural backing using properly sized fasteners rated for the rack and its anticipated load. Drywall anchors alone shall not be permitted. The vendor shall properly seal and firestop all wall, floor, and ceiling penetrations created or disturbed by its work in accordance with applicable codes and the existing fire rating of the assembly. Where an existing telecommunications grounding/bonding system is present in the closet, replacement racks shall be bonded to it. The vendor shall repair any damage to District facilities or existing equipment caused by its work at no additional cost to the District and leave work areas clean and orderly.

- Third-party equipment and cabling: Existing Zayo, Cox, and other third-party racks, fiber, cabling, and equipment are excluded from this project and shall not be disconnected, modified or reterminated without District approval.

The awarded vendor shall remain fully responsible for completion of all work. Subcontracting is permitted only with prior written approval from the District. The vendor shall identify all proposed subcontractors and the work they will perform and shall ensure that all subcontractors comply with the requirements of this RFP. The services will be awarded to one vendor and will not be separated among multiple vendors. The District reserves the right to determine which buildings to award to stay within the fiscal year budget and may award any, all, or none of the fiber runs and fiber reterminations. Vendors must provide pricing for all 14 buildings to be considered for award. Vendors that do not bid on all locations will be disqualified.

All quantities shown in this RFP are estimates for bidding and E-Rate funding purposes, may change based on actual conditions, and do not represent guaranteed quantities. The District will pay only for the actual quantities of work completed, materials installed, and services performed and accepted by the District, based on the unit prices submitted in the proposal, except that new fiber runs will be paid at the per-run price and fiber reterminations at the per-building price submitted in Attachment D. No payment will be made for estimated quantities not actually provided. Unit prices submitted in the proposal shall remain firm and shall be used to calculate District-approved additions or deductions to the quantities of materials and services included in the original scope. The vendor shall not perform work outside the approved scope or incur additional charges without prior written approval from the District. Pricing must include all hardware, labor, freight, installation, testing, documentation, and disposal. All proposal pricing shall remain firm through contract award and E-Rate funding determination unless otherwise approved in writing by the District.

Proposal Format

Vendors shall upload a PDF copy to CRW Consulting at https://www.crwconsulting.com/ifcb/ifcb-drfdetail/TkRFeU5UWXg= on or before Friday, November 27, 2026 at 11:59 pm PST.

All proposals must include the sections below. Any proposal deemed non-conforming by the Director of Technology and/or Evaluation Committee Chairman regarding format will be considered nonresponsive and will be disqualified.

Summary of Sections:

A brief explanation of each section is listed below. Vendors shall fully address each section.

- Pricing for each building, each fiber run, and fiber reterminations by building shall be listed on the pricing worksheet in Attachment D. This worksheet should be located at the front of the proposal and is mandatory.

- In addition to completing Attachment D, vendors shall provide a separate itemized pricing breakdown identifying each material or service, quantity, unit price, and extended total. Materials and labor shall be identified separately. Unit prices shall include all associated costs applicable to the item.

- Letter of Introduction – Each proposal must be accompanied by a transmittal letter that must include the following information:

- Identify the submitting organization.

- Identify the name, title, telephone number, and email address of the person or persons authorized to contractually bind the organization for this proposal.

- Be signed by a person authorized to contractually bind the organization.

- Vendor Profile – Brief history of vendor. Provide information demonstrating the vendor’s background, experience, and ability to complete the required work. Include photographs of at least three separate MDF/IDF closets from similar completed projects that demonstrate the vendor’s cable management, rack installation, patch-panel organization, labeling, and overall workmanship.

- Timeline – Please describe how this project would be implemented according to the Scope of Work above. This shall include estimated timelines.

- References – Provide at least 3 references from previous customers (school districts if possible) with similar projects in the past 5 years.

- Other Value-Added Services or Options – Vendors are encouraged to provide additional information regarding their services or industry knowledge that will support the success of the project. Vendors must provide pricing for the full required scope of work. Any optional alternatives or enhancements must be E-Rate eligible and shall be clearly identified and priced separately from the required work.

- Provide a sample contract or agreement for the District’s review. Any terms or conditions that conflict with or modify the requirements of this RFP must be clearly identified in the proposal and are subject to District approval.

Insurance Requirements

The awarded vendor shall maintain commercial general liability and workers’ compensation insurance throughout the project and shall provide proof of coverage to the District upon request.

Confidential Material

Any material that is to be considered confidential in nature must be clearly marked as such and will be treated as confidential by the District to the extent allowable by law.

Questions and Answers

All questions regarding the RFP must be directed to CRW Consulting at https://www.crwconsulting.com/ifcb/ifcb-drfdetail/TkRFeU5UWXg= by Wednesday, October 28, 2026 at 11:59 pm CST. Questions limited to walkthrough scheduling may be directed to the District Technology Director. All vendor questions and the District’s answers and RFP updates will be available to all respondents by way of a Q&A page on the District’s consultant website located at https://www.crwconsulting.com/ifcb/ifcb-drfdetail/TkRFeU5UWXg= .

Evaluation Process

A District-appointed Evaluation Committee of at least three (3) members will assess the relative merits of proposals and determine the proposal most advantageous to the District. The price of E-Rate-eligible products and services shall be weighed more heavily than any other single evaluation factor. Proposals may be accepted and evaluated without discussion, and vendors shall not initiate such discussions. If fewer than five proposals are submitted, the Committee may recommend an award or reissue the RFP.

Following evaluation, the Committee will select a finalist, and the Technology Director will notify that vendor. The Committee will recommend for award the vendor whose proposal is determined to be most advantageous to the District. The District will then proceed with contract negotiations.

The USD 261 Board of Education has final authority in this matter. Any contract resulting from this RFP is subject to final approval by the Board, which reserves the right to reject any or all proposals.

Schedule of Events

| 1. | Wednesday, October 7, 2026: | This RFP is being issued by the District and will be posted on https://www.crwconsulting.com/ifcb/ifcb-drfdetail/TkRFeU5UWXg= |
| --- | --- | --- |
| 2. | Wednesday, October 21, 2026 at 9:00 am CST: | Walkthrough #1 located at The Learning Center at 150 Stewart, Haysville, KS 67060 |
| 3. | Tuesday, October 27, 2026 at 2:00 pm CST: | Walkthrough #2 located at The Learning Center at 150 Stewart, Haysville, KS 67060 |
| 4. | Wednesday, October 28, 2026 at 11:59 pm CST: | Response deadline to written questions/RFP Amendments. No questions will be answered afterward. |
| 5. | Friday, November 27, 2026 at 11:59 pm CST: | ALL PROPOSALS MUST BE UPLOADED AT https://www.crwconsulting.com/ifcb/ifcb-drfdetail/TkRFeU5UWXg= FOR REVIEW AND EVALUATION BY THE TECHNOLOGY DIRECTOR FOR DISTRIBUTION TO THE EVALUATION COMMITTEE. Proposals received after this deadline will not be accepted. |

Thank you for considering this request for proposal. Do not hesitate to contact CRW Consulting at https://www.crwconsulting.com/ifcb/ifcb-drfdetail/TkRFeU5UWXg= if you have questions or need additional information.

Attachment A – Building Locations

| Building | Abbreviation | Address |
| --- | --- | --- |
| Administration Building | ADM | 1745 W. Grand Ave., Haysville, KS 67060 |
| Campus High School | CHS | 2100 W. 55th St. S., Wichita, KS 67217 |
| Freeman Elementary School | FES | 7303 S. Meridian, Haysville, KS 67060 |
| Grand Lane | GL | 114 Stewart, Haysville, KS 67060 |
| Haysville High School / Tri-City Day School | HHS/TCDS | 1975 W. Grand Ave., Haysville, KS 67060 |
| Haysville Middle School | HMS | 900 W. Grand Ave., Haysville, KS 67060 |
| Haysville West Middle School | HWMS | 1956 W. Grand Ave., Haysville, KS 67060 |
| Nelson Elementary School | NES | 245 N. Delos, Haysville, KS 67060 |
| Oatville Elementary School | OES | 4335 S. Hoover, Wichita, KS 67215 |
| Operations Building | OPS | 1745 W. Grand Ave., Haysville, KS 67060 (driveway next to water tower) |
| Prairie Elementary School | PES | 7101 S. Meridian, Haysville, KS 67060 |
| Rex Elementary School | RES | 1100 W. Grand Ave., Haysville, KS 67060 |
| Ruth Clark Elementary School | RCES | 1900 W. 55th St. S., Wichita, KS 67217 |
| The Learning Center | TLC | 150 Stewart, Haysville, KS 67060 |

Attachment B – Cat5/6 Retermination, Patch Panel & Rack Breakdown by Closet

Summary

| Description | Qty |
| --- | --- |
| 24-port modular patch panels | 215 |
| 4u – vertical | 3 |
| 6u – vertical | 2 |
| 12u | 24 |
| 18u | 12 |
| 26u | 4 |
| 42u | 3 |
| Estimated reterminations in closets | 3,069 |
| Estimated reterminations in classrooms, offices, etc | 2,075 |

Terminations, Patch Panels & Racks Needed Breakdown by MDF/IDF

| Building | Location | # on Map | Closet Reterminations | Remote End-Reterminations | 24-Port Patch Panel Total | Rack Model | Rack Qty |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ADM | Server Room | 2 | 13 | 5 | 2 | 42u | 1 |
| ADM | SPED (old IT) | 3 | 38 | 32 | 2 | 4u – vertical | 1 |
| CHS | Main Office | 1 | 99 | 68 | 7 | 42u | 1 |
| CHS | F | 2 | 169 | 153 | 10 | 18u | 1 |
| CHS | E | 3 | 48 | 25 | 4 | 12u | 1 |
| CHS | D | 4 | 107 | 86 | 7 | 18u | 1 |
| CHS | B | 5 | 58 | 47 | 3 | 12u | 1 |
| CHS | A | 6 | 130 | 113 | 8 | 18u | 1 |
| CHS | I | 7 | 51 | 32 | 4 | 12u | 1 |
| CHS | Print Shop | 8 | 19 | 8 | 1 | 12u | 1 |
| CHS | H | 9 | 46 | 20 | 4 | 18u | 1 |
| CHS | G | 10 | 80 | 49 | 6 | 18u | 1 |
| CHS | C | 11 | 58 | 47 | 3 | 12u | 1 |
| FES | Library (C20) | 1 | 90 | 71 | 6 | 18u | 1 |
| FES | Cafeteria (D11) | 2 | 86 | 62 | 6 | 18u | 1 |
| FES | B Closet | 3 | 38 | 27 | 2 | 12u | 1 |
| GL | IT (old HHS) | 1 | 91 | 83 | 6 | 12u | 2 |
| GL | SBHC Office (old Lab) | 2 | 19 | 18 | 1 | 12u | 1 |
| GL | PAT (old PS) | 3 | 19 | 18 | 1 | 12u | 1 |
| HMS | Cafeteria | 1 | 33 | 15 | 3 | 18u | 1 |
| HMS | C | 2 | 141 | 115 | 9 | 18u | 1 |
| HMS | B8 (boiler room) | 3 | 86 | 62 | 6 | 12u | 2 |
| HMS | B | 4 | 46 | 20 | 4 | 6u – vertical | 2 |
| HMS | H16 | 5 | 51 | 30 | 4 | 12u | 1 |
| HMS | J9 | 6 | 50 | 29 | 4 | 12u | 1 |
| HWMS | Cafeteria | 1 | 121 | 78 | 8 | 26u | 1 |
| HWMS | Library | 2 | 122 | 80 | 8 | 26u | 1 |
| HHS/TCDS | Room 187 | 1 | 137 | 92 | 9 | 26u | 1 |
| HHS/TCDS | Room 141 | 2 | 74 | 33 | 7 | 18u | 1 |
| NES | C | 1 | 106 | 70 | 7 | 12u | 2 |
| NES | A | 2 | 123 | 87 | 8 | 18u | 1 |
| OES | Stock Room | 1 | 116 | 62 | 8 | 18u | 1 |
| OES | New Janitor Closet | 2 | 36 | 26 | 3 | 4u – vertical | 2 |
| OPS | Boiler Room | 1 | 31 | 18 | 3 | 12u | 1 |
| PES | Office | 1 | 109 | 21 | 9 | 26u | 1 |
| RES | C | 1 | 107 | 73 | 7 | 12u | 2 |
| RES | A | 2 | 70 | 39 | 5 | 12u | 1 |
| RCES | Square | 1 | 72 | 55 | 5 | 12u | 1 |
| RCES | Triangle | 2 | 54 | 40 | 4 | 12u | 1 |
| RCES | Circle | 3 | 31 | 14 | 3 | 12u | 1 |
| RCES | Office | 5 | 44 | 19 | 4 | 12u | 1 |
| TLC | TLC | 1 | 50 | 33 | 4 | 42u | 1 |
| Totals |  |  | 3,069 | 2,075 | 215 |  | 48 |

Attachment C – New Fiber Runs & Reterminations

New Fiber Runs

| Building | Description | Closet # From & To | #/Color on Map | Est. Feet |
| --- | --- | --- | --- | --- |
| CHS | A Hall | 6-1 | Brown | 850 |
| CHS | B Hall | 5-1 | Purple | 850 |
| CHS | C Hall | 11-1 | Blue | 750 |
| CHS | D Hall | 4-1 | Green | 600 |
| CHS | E Hall | 3-1 | Red | 400 |
| CHS | F Hall | 2-1 | Orange | 700 |
| CHS | G Hall | 10-1 | White | 600 |
| CHS | H Hall | 9-1 | Orange | 450 |
| CHS | I Hall | 7-1 | Black | 800 |
| HMS | Boiler Room | 3-1 | Brown | 400 |
| HMS | B Hall | 4-1 | Orange | 250 |
| HMS | C Hall | 2-1 | Purple | 250 |
| HMS | H Hall | 5-1 | Green | 300 |
| HMS | J Hall | 6-1 | Yellow & Green | 600 |
| HWMS | Closet 1 to 2 | 1-2 | Red | 550 |

Fiber Reterminations

| Building | Reterminations | Closet | Notes |
| --- | --- | --- | --- |
| ADM | 24 | 1 | 12 SM to Freeman, 12 SM to Operations |
| CHS | 12 | 10 | 12 SM to tennis court |
| FES | 12 | 1 | 12 SM to Admin |
| HHS/TCDS | 24 | 1 | 12 SM to Operations 12 OM3 to Closet 2 |
| HHS/TCDS | 12 | 2 | 12 OM3 to Closet 1 |
| HMS | 24 | 1 | 12 SM to Rex and 12 OM3 to Ticket Booth |
| HMS | 12 | 3 | 12 SM Boiler to S Gym |
| HWMS | 24 | 1 | 24 SM Concessions and Greenhouse |
| NES | 12 | 2 | 12 SM to Hemphill |
| OPS | 24 | 1 | 24 SM Admin/HHS |
| RES | 36 | 1 | 12 SM to HMS 24 OM3 to Stadium |
| Totals | 216 |  |  |

Attachment D – Pricing Worksheet

Proposal Pricing: Pricing shall be entered into the applicable tables below: (1) Cat5/6 Closet Work Pricing, including parts, labor, and total cost; (2) New Fiber Runs Pricing, priced per run; and (3) Fiber Reterminations Pricing, priced by building. Lump-sum pricing without this detail will not be accepted. This worksheet should be located at the front of the proposal. Yellow shaded cells are to be completed by the vendor.

| Scope | Total Cost |
| --- | --- |
| Cat5/6 Closet Work |  |
| New Fiber Runs |  |
| Fiber Reterminations |  |
| TOTAL PROPOSAL COST (all closet work, fiber runs & fiber reterminations) |  |

Cat5/6 Closet Work Pricing

| Building | Parts | Labor | Total |
| --- | --- | --- | --- |
| Administration Building |  |  |  |
| Campus High School |  |  |  |
| Freeman Elementary School |  |  |  |
| Grand Lane |  |  |  |
| Haysville High School / Tri-City Day School |  |  |  |
| Haysville Middle School |  |  |  |
| Haysville West Middle School |  |  |  |
| Nelson Elementary School |  |  |  |
| Oatville Elementary School |  |  |  |
| Operations Building |  |  |  |
| Prairie Elementary School |  |  |  |
| Rex Elementary School |  |  |  |
| Ruth Clark Elementary School |  |  |  |
| The Learning Center |  |  |  |
| Optional – Vendor’s discount if the district decides to award the contract for all listed locations. |  |  |  |
| TOTAL BUILDING/CLOSET WORK |  |  |  |

New Fiber Runs Pricing

| Building | Description | Closet # From & To | Est. Feet | Total Cost |
| --- | --- | --- | --- | --- |
| Campus High School | A Hall | 6-1 | 850 |  |
| Campus High School | B Hall | 5-1 | 850 |  |
| Campus High School | C Hall | 11-1 | 750 |  |
| Campus High School | D Hall | 4-1 | 600 |  |
| Campus High School | E Hall | 3-1 | 400 |  |
| Campus High School | F Hall | 2-1 | 700 |  |
| Campus High School | G Hall | 10-1 | 600 |  |
| Campus High School | H Hall | 9-1 | 450 |  |
| Campus High School | I Hall | 7-1 | 800 |  |
| Haysville Middle School | Boiler Room | 3-1 | 400 |  |
| Haysville Middle School | B Hall | 4-1 | 250 |  |
| Haysville Middle School | C Hall | 2-1 | 250 |  |
| Haysville Middle School | H Hall | 5-1 | 300 |  |
| Haysville Middle School | J Hall | 6-1 | 600 |  |
| Haysville West Middle School | Closet 1 to 2 | 1-2 | 550 |  |
| TOTAL COST FOR NEW FIBER RUNS |  |  | 8,350 |  |

Fiber Reterminations Pricing

| Building | Reterminations | Total Cost |
| --- | --- | --- |
| Administration Building | 24 |  |
| Campus High School | 12 |  |
| Freeman Elementary School | 12 |  |
| Haysville High School / Tri-City Day School | 36 |  |
| Haysville Middle School | 36 |  |
| Haysville West Middle School | 24 |  |
| Nelson Elementary School | 12 |  |
| Operations Building | 24 |  |
| Rex Elementary School | 36 |  |
| TOTAL COST FOR FIBER RETERMINATIONS | 216 |  |

Attachment E – District Part Specifications

| Item | Description |
| --- | --- |
| Rack – Wall Mount | 24” depth (when possible), not enclosed, square hole, Rackstuds preferred |
| Rack – Floor Mount | 4 post, 43” depth |
| Patch Panel – Cat5/6 | Modular, 24-port, Keystone compatible, no coupler style |
| Patch Cable – Cat5/6 | Slim 28 AWG Cat6 patch cables rated for PoE Type 4 |
| Patch Panel – Fiber | All LC |
| Patch Cable – Fiber | Single Mode |
| Single Mode Fiber Cable | 12-strand, OS2 single-mode fiber (9/125 µm) |
| Wall Plate Jacks | Keystone compatible |
| Wall Plates | Keystone compatible |