# REQUEST FOR PROPOSAL (RFP)

## E-RATE FUNDING YEAR 2027–2028

### CATEGORY 2: NETWORK EQUIPMENT, FIREWALL, INSTALLATION, AND SETUP SERVICES

RFP Identifier: FY2027-C2-EQUIP-01

FCC Form 470 Application Number: 270002115

Applicant Name: HANNIBAL SCHOOL DISTRICT 60

Billed Entity Number (BEN): 136960

Issue Date: October 22, 2026

Questions Due Date: November 9, 2026 to John Watkins at jwatkins@hannibal60.com

Bid Submission Deadline: November 19, 2026 to John Watkins at jwatkins@hannibal60.com

## 1. STATEMENT OF PURPOSE & SCOPE OF WORK

The Applicant is seeking competitive proposals for E-Rate Category 2 eligible internal connections equipment, licensing, installation, and setup services for Funding Year 2027–2028 (July 1, 2027 – June 30, 2028).

The objective of this procurement is to upgrade campus core, aggregation, and access network switching alongside the primary perimeter firewall infrastructure.

### 1.1 Technical Architecture Overview

- Switching Infrastructure: Desired baseline architecture utilizes current-generation HPE Aruba enterprise switching hardware (or equivalent) delivering:

  - Minimum 24-port SFP+ (1G/10G) fiber/copper aggregation ports.

  - Minimum 120-port Gigabit Ethernet (10/100/1000Base-T) RJ-45 access ports with PoE+ (IEEE 802.3at/802.3bt).

  - Built-in high availability and hardware redundancy (dual hot-swappable power supplies, redundant fans, and virtual chassis/stacking capabilities).

- Perimeter Security Appliance: Desired baseline solution utilizes a Fortinet FortiGate security appliance (or equivalent) capable of delivering a minimum of 5 Gbps Threat Protection / NGFW throughput with full security inspection enabled.

- Services: Professional turnkey installation, physical mounting, structured cable patch connections, logical configuration, failover testing, system turn-up, and documentation.

### 1.2 Vendor Scope & Single-Vendor Preference

To ensure seamless interoperability, single-point accountability, and streamlined project management during deployment, preference will be given to vendors capable of supplying 100% of the requested hardware, software, licensing, and professional installation services under a single award.

## 2. GENERAL E-RATE INSTRUCTIONS & COMPLIANCE

- E-Rate Eligibility: All requested equipment and services must be eligible under USAC E-Rate Category 2 guidelines for FY2027–2028. Any non-eligible hardware, software, or service components must be clearly identified and cost-allocated separately in the bid response.

- SPIN Requirement: Bidders must possess a valid, active Service Provider Identification Number (SPIN) and have an updated FCC Form 473 (Service Provider Annual Certification) on file with USAC.

- Equivalent Products ("Or Equivalent"): Manufacturer names, model numbers, and part numbers listed in this document indicate desired minimum standards of quality, feature sets, and performance. Bidders may propose functionally equivalent hardware/software provided they clearly prove parity in feature capabilities, management, and performance.

- Lowest Corresponding Price (LCP): Bidders must comply with the FCC’s LCP rule, guaranteeing that prices offered are no higher than the lowest price charged to non-E-Rate customers for similar services.

- Funding Contingency: Any contract resulting from this RFP is contingent upon E-Rate funding approval from the Universal Service Administrative Company (USAC) and authorization by the local governing board.

## 3. TECHNICAL & HARDWARE SPECIFICATIONS

### Section A: Enterprise Switching Solution (HPE Aruba or Equivalent)

The switching architecture must be current-generation, enterprise-grade hardware and provide the following total port densities and functional performance across the network:

#### A1. Fiber Aggregation / Core Switch (24- port SFP+)

- Port Configuration: Minimum 24 X 1G/10G SFP+ optical/copper ports.

- Performance: Non-blocking switching fabric supporting wire-speed L2/L3 forwarding.

- High Availability & Redundancy:

  - Dual, hot-swappable AC power supplies  power redundancy.

  - Redundant, field-replaceable fan modules.

  - Hardware support for Virtual Switching Extension (VSX) or Virtual Switching Framework (VSF) for hitless control-plane failover and multi-chassis link aggregation (M-LAG).

- Management & Features: Full Layer 3 protocol suite (OSPF, BGP, Static Routing, VRF), telemetry, automated network analytics, and cloud/centralized management integration.

- Reference Equipment: HPE Aruba CX 6300M 24SFP+ Switch (JL658A / S0G03A) or equivalent.

#### A2. Access Switches (120-port PoE+ Gigabit )

- Port Configuration: Minimum 120 x 10/100/1000Base-T PoE+ ports (typically satisfied via 3X 48-Port switches or equivalent chassis configuration).

- Power over Ethernet: IEEE 802.3at PoE+ (minimum 30W per port) with sufficient power budget to support high-density Wi-Fi 6E/7 access points and IP surveillance.

- Uplinks & Stacking: Minimum 4x 10G/25G SFP+ uplink ports per switch unit, with hardware stacking backplane support.

- Redundancy: Modular secondary hot-swappable power supply support for redundant power and PoE power expansion.

- Reference Equipment: HPE Aruba CX 6300M or 6200M 48-Port Class 4/6 PoE+ Switches (or equivalent).

#### A3. Optical Transceivers & Interconnects

- Provide all necessary 10G SFP+ SR optical transceivers and Direct Attach Copper (DAC) cables to link access switches to aggregation switches and firewall appliances.

### Section B: Next-Generation Firewall Solution (Fortinet FortiGate or Equivalent)

The security appliance must deliver enterprise-grade threat protection and core routing capabilities at the network perimeter.

#### B1. Security Appliance Requirements

- Throughput Standards: Minimum 5 Gbps Threat Protection / NGFW Throughput with Application Control, Intrusion Prevention (IPS), Antivirus, and SSL/TLS Deep Packet Inspection active.

- Network Interfaces:

  - Minimum 2x 10GbE SFP+ WAN/LAN ports.

  - Minimum 8x 1GbE RJ-45 LAN ports.

- High Availability: Hardware support for Active-Passive and Active-Active stateful failover configurations.

- Redundancy: Dual internal AC power supplies.

- Software & Subscriptions: Include 1-Year (or multi-year matching contract term) Unified Threat Protection (UTP) or Enterprise Security Subscription including 24x7 vendor support, firmware updates, IPS, Antivirus, Web Filtering, and Application Control.

- Reference Equipment: Fortinet FortiGate 200F / 400F Series (or current-generation equivalent model meeting or exceeding 5 Gbps Threat Protection throughput).

### Section C: Installation, Configuration, and Setup Services

The vendor must provide complete turnkey implementation services executed by vendor-certified engineers (e.g., Aruba ACMP/ACDX, Fortinet NSE 4/7 or equivalent).

- Physical Deployment:

  - Unpack, inventory, rack-mount, and ground all switches and firewall appliances in designated equipment racks.

  - Neatly dress copper and fiber patch cords, install transceivers, and ensure proper cable management.

- Logical Configuration:

  - Configure Layer 2 VLANs, standard Spanning Tree protocols (RSTP/MSTP), L3 subnets, dynamic routing protocols, and stacking/HA cluster topologies.

  - Configure Firewall WAN settings, NAT rules, Security Policies, Content Filtering, IPS profiles, and SSL VPN/IPsec remote access tunnels.

  - Integrate administrative access with centralized authentication (RADIUS/TACACS+/LDAP).

- Testing & Commissioning:

  - Perform failover validation (power supply unplug tests, stacking link cut tests, firewall HA failover).

  - Perform throughput verification and internet connectivity testing.

- Documentation & Handoff:

  - Supply complete "As-Built" network topology diagrams, port mapping tables, configuration backup files, and serial number inventory lists.

  - Conduct a 2-hour administration walkthrough and handoff session with District IT staff.

## 4. PRICING & BID FORM

Bidders must present itemized pricing similar to the format below. Separate E-Rate Eligible costs from Ineligible costs.

| Line | Qty | Part Number / Model | Detailed Description | Unit Price | Total Eligible Cost | Ineligible Cost | E-Rate Eligible (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | JL658A (or equiv) | Aruba CX 6300M 24p SFP+ Aggregation Switch | $ |  |  |  |
| 2 | 3 | [48-Port Model] | Aruba CX 48G PoE+ Access Switch (or equiv) | $ |  |  |  |
| 3 | X | [PSU / Accessories] | Secondary Redundant PSUs & Stacking Modules | $ |  |  |  |
| 4 | X | [Optics / Cables] | 10G SFP+ Transceivers / DAC Uplink Cables | $ |  |  |  |
| 5 | 1 | FG-200F (or equiv) | FortiGate Next-Gen Firewall (5 Gbps Threat) | $ |  |  |  |
| 6 | 1 | [License / Support] | FortiCare 24x7 & FortiGuard UTP Subscription (or equiv) | $ |  |  |  |
| 7 | 1 | [Service Fee] | Turnkey Professional Installation & Setup | $ |  |  |  |
| TOTAL |  |  |  |  | $ | $ |  |

## 5. EVALUATION CRITERIA AND MATRIX

In compliance with FCC E-Rate rules (47 CFR § 54.511), the Cost of E-Rate Eligible Products and Services will serve as the primary factor in the evaluation process, carrying the highest weight.

Bids will be evaluated according to the following point matrix:

| Criteria | Maximum Points |
| --- | --- |
| Cost of E-Rate Eligible Products and Services (Primary Factor) | 35 Points |
| Single-Vendor Turnkey Solution (Ability to supply 100% of hardware, licensing, and setup under one contract) | 20 Points |
| Technical Compliance & Hardware Performance (Compliance with the specifications and requirements identified in the RFP 5Gbps redundancy, port counts) | 15 Points |
| Vendor Qualifications, E-Rate Experience, & References | 15 Points |
| Implementation Timeline & Scope of Work Plan | 10 Points |
| Ineligible Cost | 5 Points |
| TOTAL SCORE | 100 Points |

## 6. PROPOSAL SUBMISSION INSTRUCTIONS

- Submissions: Bids must be submitted via email in PDF format to jwatkins@hannibal60.com.

- Subject Line Format: FY2027 Category 2 Proposal - [Vendor Name] - RFP FY2027-C2-EQUIP-01

- Deadline: All proposals must be received no later than November 19, 2026.

- Required Enclosures:

  - Itemized Pricing Table (Section 4).

  - Valid Service Provider Identification Number (SPIN) & Form 473 verification.

  - Three (3) K-12 or enterprise references for similar scope deployments.

  - Proposed Project Plan & Scope of Work timeline.

## 7. VENDOR CERTIFICATION FORM

(To be signed and returned with the vendor's response)

Vendor Name: ____________________________________________________

SPIN Number: ____________________________________________________

Authorized Representative Name: ______________________________________

Title: _________________________________________________________

Signature: ___________________________________ Date: _______________