Purpose

The West Branch Local School District is seeking installation of one fiber circuit between the Wellness Center and the Stadium Press Box.

A purchase order to execute this RFP will not be issued until positive E-Rate funding decision has been received.

Technical Requirements

- The fiber shall be a 6-strand single mode, indoor/outdoor, tight buffered, plenum rated cable.

- The length of the fiber run is approximately 600 feet.  However, the vendor is solely responsible for determining the amount of fiber needed.

- Five Hundred feet of fiber will be installed in existing conduit.  The final 100 feet will be above ground inside metal conduit.

- The fiber shall be terminated in the Wellness Center’s MDF in an existing Meraki MS225 switch.

- Connectors and SFP’s shall be provided and installed to provide transmission bandwidth of 10 Gbps.

- The following equipment shall be provided and installed in the press box

  - 12U Rack

  - Fiber Splice panel

  - 24 port Cat 6 passthrough patch panel

  - 750 watt battery backup

  - Meraki 48port MS225FP comparable Full POE+ switch

NOTE: Two needed 10GB fiber patch cables will be provided by the district

- General Requirements

  - All equipment shall be new and shall include manufacturer warranty (three year) that includes next day replacement and all software updates.

- Installation and Configuration

  - Installation shall include all needed labor, equipment and miscellaneous material (e.g.: conduit) needed to make the fiber and equipment fully functional.

  - Installation shall be completed no later than six months from the date of the purchase order.

  - Installation activities shall occur during non-school hours unless prior arrangements are made.

  - Bidders will be required to provide proof of meeting minimum insurance requirements.

Acceptance Requirements

Acceptance will be based on the vendor demonstrating that fiber operating properly.

Billing Requirements

The school shall not be billed until the above Acceptance Requirements have been met.

Site Visit

An on-site visit is required before our acceptance of your RFP response.  Contact Jim Graham at 330-938-4411 (jim.graham@wbwarriors.org ) to schedule a site visit.

Vendor Submission Requirements

Vendors wishing to be considered shall provide a proposal that meets the following requirements:

- Submissions will be accepted in electronic format only

- Proposals are to be submitted to grantgermann@erate-edplus.com

- Proposals must be submitted no later than 28 days after the issuance of the Form 470.  The applicant reserves the right to reject bids that are received after this date.

- Submissions shall include the following:

  - The bid shall clearly show for each item proposed: Manufacturer, Manufacturer Part#/SKU; Unit Cost.

  - Name of two current or previous clients where fiber installation was part of the service.  For each client the following information shall be provided:

- Client Name

- Client Point of Contact

- Contact information (e-mail/phone number)

  - General information about the vendor.  The intent is to familiarize the school with the vendor’s background such as the number of employees, annual revenue, the types of clients served by the vendor, and the types of services and technologies provided by the vendor.

  - Required Vendor Information:

- The vendor’s name as listed by USAC

- Vendor’s E-Rate SPIN number or provide documentation that a SPIN has been applied for

- Website address,

- The name and contact information of the individual that is coordinating the vendor’s submission.

Summary

We reserve the right to reject any or all submissions, to extend the submission deadline, and to select a vendor in a manner that provides the best value for the school.  We do not expressly state or imply any obligation to reimburse the responding vendors for any expenses in preparing submissions in response to this request.

Questions

Any questions related to this RFP should be addressed by email to the individual that proposals are submitted no later than one week prior to the Allowable Contract Date.